# Sun Vector Azimuth and Elevation Computation

# Scope

First draft version - meant for internal use only.

# Introduction

The position of the sun is important for several reasons. For one, the terminal should have a sun-exclusion zone to keep the optics safe. Additionally, the sun can be used for mounting offset calibration purposes. However, the computation of the sun’s position is non-trivial. While computations offline can be done directly using planetary ephemerides, these require too much static memory to be used on-board the terminal. Simplified mathematical methods can be used too, and are fit to be used on-board the terminal. However, these result in a lower precision position and still need to be converted to the terminal’s global frame to compute the azimuth and elevation angles. Therefore, in this document, the approximate computation of the sun vector and its interface with the pointing algorithm is described.

# Methods

Architecture to Determine Sun Vector’s Azimuth and Elevation

Sun Vector in Mean Of Date frame - Numerical Theory Approach

The method follows the description in [Vallado 2014, Fundamentals of Astrodynamics, Algorithm 29]. As the algorithm is originally described, the time input must be Julian Date in Universal Time (UT1). To fit in with Mynaric conversions, a time conversion is added, such that GPS Seconds can be used as the input.

Thus, the only input will be the current GPS time and the  output will be the Sun position in Mean Of Date (MOD) frame. A subsequent conversion will be needed to convert the Sun vector to Earth Centered Inertial frame.

The GPS seconds must be converted to Julian Centuries TGPS:

TGPS=JDGPS-245154536525

Where JDGPS is the Julian Date in the GPS time-scale, which can be computed as:

JDGPS=JDGPS,0+ ti,GPS/86400

Where JDGPS,0 is the Julian Date corresponding to the GPS [s] start date (1980-01-06 00:00:00 UTC, matching with GPS, later offset by number of leap seconds since 1980). ti,GPS is the number of GPS seconds at the current time. The Julian Date at GPS,0 is equal to 2444244.5.

JDUTC=JDGPS,0+ti,GPS-499-18 86400

These allow to compute the relevant Earth Equator - Sun Ecliptic plane angles in degrees:

Mean longitude of the Sun:

λS=280.46+36000.771 TGPS

Mean anomaly of the Sun: (must be reduced to 0:360 degrees)

MS= 357.5291092+35999.05034 TGPS

Approximate ecliptic longitude:

λecl= λS+ 1.914666471 * sinMS+ 0.019994643 sin2 MS

Obliquity of the ecliptic:

ϵ= 23.439291 - 0.0130042 TGPS

Which are all used to compute the normalized Earth-Sun vector in MOD:

rSun,MOD=cosλeclcosϵsinλeclsinϵsinλeclrEarth_to_Sun

Where rEarth_to_Sun is the distance from Earth to Sun in Astronomical Units and is calculated using:

rEarth_to_Sun = 1.000140612 -0.016708617cosMS- 0.000139589cos(2MS)

Finally, the Sun vector is converted to meters by multiplying it with 149597870700 [m/AU] .

Rotation of Sun Vector from MOD to ECI

A simplified IAU76 convention approach can be used to get the Precession Rotation matrix. This is used to rotate from MOD to J2000/ECI/GCRF. In the IAU76 conventions, 3 angles are set up to combine the effects of general Precession [Vallado 2014, Section 3.7.3, equations (3-88), (3-89)].

ζ= 2306.2181TTT + 0.30188TTT2+ 0.017998TTT3
Θ= 2004.3109 TTT- 0.42665 TTT2- 0.041833 TTT3
z= 2306.2181TTT+ 1.09468 TTT2+ 0.018203 TTT3

rSun,ECI=ROT3ζROT2-ΘROT1zrSun,MOD

TTT=JDTT-245154536525

JDTT=2444244.5+ ti,GPS-51.18486400

Where

DCMMOD_to_ECI=ROT3ζROT2-ΘROT1z

rSun,ECI=DCMMOD_to_ECIrSun,MOD